The instructions in the H6 Bearing Overhaul Kit are generally good and should be followed meticulously.  However, here are some SUGGESTED ADDITIONS TO THE OVERHAUL KIT INSTRUCTIONS:

UNDER WARNING;

(add)  ALL THREADS ON THE AUTOPROP ARE RIGHT HAND THREADS (CLOCKWISE TO TIGHTEN AND ANTI-CLOCKWISE TO LOOSEN)  EXCEPT

FOR THE THREADS OF THE TAB SCREW (item 4, part number H6 TRI –0221) THAT ARE LEFT HAND THREADS.

(add)  Some components will have been assembled with LockTite or other similar thread locking compound that will make removal difficult.  If the Bearing Cap cannot be removed with a two pin peg wrench due to thread locking compound, it may be necessary to fabricate a 4 pin peg wrench with the 4 each 4mm pegs (screws) on 1 inch centers.

UNDER BLADE REMOVAL ITEM 9

(add)   The ball bearing races ARE NOT PRESS INTERFERENCE FIT.  THEY ARE LOOSE FIT but may be held securely in place by dirt, debris, and accretions.  To remove the bearing races first meticulously clean every groove and recess where the bearing races rest. Then apply, and allow time to work, a good quality penetrating oil.  Small pieces of fine emery cloth may be used to further clean the race recess and between the race and the inner diameter of the race and the race recess.

Next use the sharp edge of a chisel held at a 45 degree angle and tap the center of the race groove to rotate the race in its recess.  Once the race begins to freely move in its land, turn the hub or blade over and gently tap the backside to try and dislodge the race from its land.  This is a tedious process and may take multiple applications of penetrating oil etc.  The races are case hardened steel and attempting to drill into them to tap a screw to aid in removal will be futile.  It has been suggested that super gluing a screw to the race might give some purchase in attempting to remove them.

Good Luck,  Gary Silver     s/v Liahona    Amel Super Maramu 2000,   Hull # 335